Studio 440 PLUS! from PointSource

MIDI Sample Dump - Quick Instructions

=========================== SETUP ===========================

1.  Connect a MIDI cable from the Studio 440's MIDI OUT A to

the MIDI IN jack of the other device.

2.  Connect a MIDI cable from the other device's MIDI OUT to

the MIDI IN jack of the Studio 440.

3.  Set the Studio 440's MIDI IN channel to match that of the

other device, or vice versa (don't worry about MIDI OUT

channel or MIDI mode).

4.  Make sure the sequencer is not running.

The Studio 440 is now ready to receive sample dumps from the

other device, and will also send samples out in response to

dump requests from the other device.  You don't need to call

up any specific screen.  To stop any sample transfer, hit the

STOP switch.  If using the Studio 440 with a computer, you

can ignore the rest of the quick instructions.

=============== MANUALLY-INITIATED TRANSFERS ================

Use when the "other device" is a sampler, not a computer.

Hit the SYSTEM MIDI 2 button until the "MIDI sample dump"

menu appears.

----------- TO DUMP A SAMPLE TO THE OTHER DEVICE ------------

1.  Select "Dump sample" and hit ENTER.

2.  Select the sample you wish to send (top line).

3.  (Optional)  Select a different sample number to be sent

with this sample to the other device (bottom line).

3.  Hit ENTER again.  "Sending ...." should appear in screen.

--------- TO REQUEST A SAMPLE FROM THE OTHER DEVICE ---------

1.  Select "Request sample" and hit ENTER.

2.  Select the sample number into which the Studio 440 should

store the incoming sample (top line).

3.  Select the number of the sample you want the other device

to send (bottom line).

4.  Hit ENTER again.  "Rcvng ...." should appear in screen.

Happy Dumping!

PointSource - 890412

Studio 440 PLUS! from PointSource

MIDI Sample Dump - The Long-winded Discussion

"For Those Who Love To Read"

===================== Product Overview ======================

The MIDI Sample Dump feature of the Studio 440 allows direct

digital transfer of samples via MIDI in either direction

between the Studio 440 and any other sampler or computer

which has a similar capability.  Among other possibilities,

this feature extends the power of the Studio 440 in three

significant ways:

EASY ACCESS TO SOUND LIBRARIES WHICH EXIST ON OTHER SAMPLERS.

Your Studio 440 sound library can now expand quickly by

incorporating sounds created on other samplers, without

the need to resample the sounds in an audio format.  The

highest possible fidelity is retained, and loop points

are preserved.  The Studio 440 can request samples from

other samplers via MIDI, allowing it to be used with

samplers which lack a manual-dump capability.

ACCESS TO THE POWER OF COMPUTER-BASED SAMPLE EDITING.

The Studio 440 can act as a "slave" to a computer running

one of the many fine sample editing programs now

available, allowing you to apply the greater resolution,

extended editing functions and on-screen graphical

techniques of these programs to the manipulation of your

Studio 440 sounds.

PRESERVATION OF YOUR INVESTMENT OF TIME AND EFFORT IN THE

CREATION OF YOUR Studio 440 SOUND LIBRARY.

If you choose to add another sampler to your setup, or to

upgrade to a 16-bit system, your existing Studio 440

sounds can be transferred to the new system with full

fidelity and no need to resample.

Read onward ....

PointSource - 890412

============== SDS - What It Is, What It Isn't ==============

The Studio 440 MIDI Sample Dump feature follows the

specifications of the Sample Dump Standard (SDS) section of

the Universal System Exclusive codes agreed upon by members

of the MIDI Manufacturers Association (MMA).  The SDS specs

define a communications protocol which permits the exchange

of "raw" digital sound data between any two MIDI devices,

regardless of differences between the internal sound formats

used by each device (eg, 12-bit, 16-bit, linear, companded

etc).  In addition, info on sample rate, loop points and loop

mode is exchanged via the SDS protocol.  Virtually all

samplers have enough in common to be able to use this

information, and will recognize and use the same set of SDS

messages in order to transfer it, translating it internally

as necessary.  For example, the Studio 440 can receive

samples from a 16-bit sampler and will store them as 12-bit

samples internally.  You can also send the Studio 440's

12-bit samples to a 16-bit sampler - the 16-bit machine will

store them in its own 16-bit format, although the "quality"

of the sample is still really 12 bits (so it may not sound

any better in the 16-bit machine, but on the other hand it

shouldn't sound any worse).

However, beyond the basic level of sample handling - in the

"playback control" department - samplers differ from one

another to such an extent that there is no way to include

such control information as part of the Sample Dump Standard.

(For the Studio 440, control info includes VCA, VCF and

pitch-bend envelopes, fine tuning, velocity-mod, sound names

and just about everything else which is not under the SOUND

EDIT 1 button).  This type of info, if made available, is

encoded into manufacturer-specific, device-specific System

Exclusive messages which are recognized only by another

device of the same type, or by a computer patch-editing

program designed specifically for that model.

The Studio 440 does not currently generate or recognize

System Exclusive messages of this second sort.  As a result,

some of the capabilites of "universal" sample editing

programs, such as Digidesign's Sound Designer II and Blank

Software's Alchemy, will not be applicable to the Studio 440.

For example, you won't be able to set filter cutoffs or

create Studio 440 keyboard maps using these programs, nor

translate these settings from the Studio 440 to a different

sampler via the computer - these control functions require

device-specific System Exclusive response beyond the scope of

"plain vanilla" SDS.  The "universal" editing programs

actually include special sections of software developed

specifically for each of the samplers they support.  For this

reason, the Studio 440 will not appear by name in the menus

of any of these programs.  Nevertheless, the truly universal

sample editing features of these programs will certainly work

with the Studio 440 and its sounds.

PointSource - 890412

======================== Setting Up =========================

Cable Connections

The Studio 440 performs SDS transfers with incoming messages

received on the MIDI IN jack.  This should be connected to

MIDI OUT on the other device.  Outgoing messages are sent via

the MIDI OUT A jack.  This should be connected to MIDI IN on

the other device.  The MIDI OUT B jack and the TERMINAL/SYNC

jacks are not used for Studio 440 SDS transfers.

Because of the way SDS works, it is best to use two MIDI

cables and make both input and output connections to the

appropriate jacks on the other device, even if you plan to

transfer samples only in one direction.  Doing this allows

for "closed loop" communication between the two devices and

will result in faster transfers and smoother operation.  In

fact, it is necessary to use the two-cable connection if you

wish to use the dump request capability of either device.

One example of this situation is using the Studio 440 with a

computer-based sample editing program.

However, if two-cable hookup is inconvenient, transfers can

be made with a single cable connected appropriately for the

desired transfer direction.  For this to work, it must be

possible to initiate a sample dump manually on whichever

device is the transmitter (the Studio 440 fully supports this

mode of operation as well).

Channel and Other Settings

The MIDI channel number set for the Studio 440 must match

that of the other device.  The Studio 440 ignores any and all

SDS messages which do not appear on its selected channel.

This is the IN:  channel which is set in the CHANNELS screen

under the SYSTEM MIDI 1 button.  This channel is always in

effect for SDS transfers, and applies to both incoming and

outgoing SDS messages - the OUT:  channel set in the same

screen is not used by the Studio 440 for SDS transfers, and

setting it to "OFF" will not affect SDS operation.  Other

MIDI settings on the Studio 440, such as "ECHO", "IN TO",

"MTC" and "SOUND PERFORM MIDI MODE", also have no effect on

SDS operation.

Note that while most MIDI messages are confined to 16

channels, the SDS spec allows for up to 127 channels.  Some

devices allow you to use the full range of channel numbers

for SDS transfers.  Because channel numbers above 16 are not

allowed for "standard" MIDI messages, these devices will

have a separate setting for the SDS channel number, which may

be called something like "device ID".  If present, this is

the channel which must match that of the Studio 440.  The

Studio 440 performs SDS transfers on channels 1-16 only.

PointSource - 890412

================= Remote Control Operation ==================

Normally, the Studio 440 is ready to receive a sample dump or

respond to a dump request at any time - all you need to do is

send one on in.  This allows it to be used easily with a

computer acting as the "master" device.  The main exception

to this is that sample dumps and dump requests are ignored by

the Studio 440 if the sequencer is running - incoming SDS

messages will not cause the sequencer to stop, and may even

get recorded into a sequence if RECORD is active at the time

(this is NOT recommended, by the way).  Also, sample/sequence

editing operations and disk operations usually lock out MIDI

response and will prevent acknowledgement of these messages.

The dump header - the first part of any sample dump - tells

the Studio 440 what it needs to know about the sample being

sent to it.  Most importantly, it indicates the size of the

sample, and the sample number under which the new sample

should be stored.  If there is not enough memory available in

the 440 to store a sample of the indicated size, the 440 will

send a message back to the transmitting device in an attempt

to halt the incoming dump, and will ignore the dump if it

comes in anyway (as might occur if the two-cable connection

is not used).  A message will also appear in the display:

����������������Ŀ

�Sample too large�

�- unable to rcv �

������������������

In addition, if the indicated sample number is already in use

in the Studio 440, then the existing sample will have to be

deleted before the new sample can be stored there.  In this

case, the memory which would be freed by deleting the

existing sample is taken into account when deciding if there

is enough memory space to receive the new sound.  If not, the

existing sound is not deleted, and the dump is rejected as

discussed above.  Otherwise, the Studio 440 automatically

deletes the sound, after first sending out a message telling

the sender to wait.  This message appears:

����������������Ŀ

�Target sound is �

�being deleted...�

������������������

and may remain for as long as several seconds.  After this,

----------- Remote Control Operation (continued) ------------

data transfer should begin.  The following screen lets you

know exactly what is happening:

����������������Ŀ

�Rcvng sample #12�

�103472 wds to go�

������������������

The number on the bottom line indicates the number of sample

words the Studio 440 is still expecting to receive.

Normally, you will see it going through a continuous rapid

countdown.  If this number is not changing, it means that the

transmitting device is not sending anything to the Studio

440.  With some computers and other devices which may need to

fetch sample data from disk during a transfer, it will be

normal to see the countdown go through an occasional brief

pause.  When the countdown goes to zero, you'll see this

message:

����������������Ŀ

�Sample transfer �

�  is complete   �

������������������

This tells you that the Studio 440 is back in your hands and

ready to resume normal operation.  At any time before the

countdown reaches zero, you can abort the transfer by hitting

the STOP switch (on the Studio 440 end, at least - the

transmitting device may not get the message, in which case it

will keep on sending).  If you do this, you may want to

delete the partially-received sample before continuing, to

avoid wasted memory space.

Response to a received dump request message, apart from being

completely automatic, is very similar to operation during a

manually-initiated dump.  See the section below entitled

"Function Screen 2 - Dumping Samples".

See also the section below entitled "Sample Receive - Further

Details" for more info on how the Studio 440 receives

samples.

================= The SDS Function Screens ==================

The function screens related to SDS operation are mostly

useful for exchanging samples between the Studio 440 and

another sampler.  When doing this, the Studio 440 can act as

the "master" and control the other sampler.  If you are

working with a computer, you shouldn't need to deal with

these screens and functions - the Studio 440 is set up to

work correctly with a computer when you turn it on.  Also,

most computers will not respond to transfers initiated by the

Studio 440 - the computer normally expects to be the master.

The SDS function screens appear under the SYSTEM MIDI 2

button.  You will initially see a single screen which looks

like this:

����������������Ŀ

�MIDI sample dump�

�     Set options�

������������������

This screen is actually a function menu.  The bottom line can

be adjusted using the data slider or the INC/DEC keys to read

"Set options", "Dump sample" or "Request sample".  Hit the

ENTER key when you have selected the function you want, and

you will obtain the associated function screen.  The

individual function screens are discussed below.

========== Function Screen 1 - Sample Dump Options ==========

Currently there is only one option, related to receiving

sample dumps.  It's set in the following screen:

����������������Ŀ

�Rcv samp number:�

�from dump header�

������������������

This option determines where the Studio 440 gets the sample

number under which it stores a received sample.  The screen

above shows the default setting - the sample number is

obtained from the dump header of the incoming sample.  This

is the setting you will normally want to use.  Especially

when using the Studio 440 with a computer-based sample

editor, it's simplest to have the 440 act as a "slave" to the

device which is sending it samples, and let the sending

device be the "master" as far as controlling where its

samples will go in the 440.  It's also what computers expect.

On the other hand, when receiving samples from another

sampler, it's likely that the sample numbers which that

sampler sends will not match the numbering scheme you want to

use in the 440 - you may want to decide this yourself on a

sample-by-sample basis.  In this case, you'd use the other

setting for this option:

����������������Ŀ

�Rcv samp number:�

�from current snd�

������������������

This indicates that the sample number in a received dump

header will be ignored, and that any received sample will be

stored under the "current" sound number.  Normally the

current sound is the one that you played last, or whose

number is displayed in one of the SOUND EDIT screens.

(However, before you jump to this setting, be sure to read

the section below on requesting samples from another

sampler - this setting does not apply if you use the

"Request sample" function of the Studio 440).

============ Function Screen 2 - Dumping Samples ============

If you select "Dump sample" in the SDS menu and then hit the

ENTER key, you'll see a screen which looks like this:

����������������Ŀ

� COWBELCH   #12 �

�send out as # 12�

������������������

On the upper line you can select the sound to be sent out

when the ENTER key is hit - the name of the sound appears

alongside the sound number.  This is the Studio 440's

"current" sound - thus, in addition to the data slider,

decimal and INC/DEC keys, you can select this sound by

playing it on the pads.  Also note that the ALT PARAMS key

has an effect here - it will select the set of START/END and

LOOP points to be used when the sound is transmitted.  The

Studio 440 does not blindly transmit the entire sample, but

instead sends the portion of it which is indicated by the

currently active set of START/END points.

On the bottom line you can select the sample number to be

sent with this sample.  Any value in the range 1-256 can be

used, since some samplers allow you to have many more than 32

samples.  This number is inserted into the dump header

message which is the first part of every outgoing sample dump

- it tells the receiving sampler which sound number to assign

to the incoming sample.  In general, by using this control,

you can send Studio 440 samples exactly where you want them

to go in the receiving sampler, without having to touch any

of its controls (not always, however, since some samplers

ignore this number when receiving sample dumps).  Note that

whenever a different sample is selected for transmission,

this number is automatically changed to match that of the

selected sample - thus, in many cases, you won't need to

specifically alter it.

Hit the ENTER key to start sending.  You'll see a screen

which looks like this:

����������������Ŀ

�Sending COWBELCH�

�100440 wds to go�

������������������

The number on the bottom line counts down to zero to indicate

the progress of the transfer.  A completion message will

appear when it reaches zero.  The countdown may pause one or

more times during a transfer if the receiving device needs to

allocate memory for the incoming sample or transfer data to

disk.  Also, the Studio 440 will wait up to two seconds for a

------ Function Screen 2 - Dumping Samples (continued) ------

response message from the receiving device after sending the

dump header (which happens when you hit ENTER).  If you have

correctly set up the two-cable connection, the transfer

countdown will usually begin almost immediately - if you

always get the two-second delay, you may want to recheck your

connections.

At any time before the countdown reaches zero, you can abort

the transfer by hitting the STOP switch.  The Studio 440 will

send a message to the receiving device which in most cases

will convince it to stop listening and go back to standby.

Sample transmit in response to a received dump request

message is almost identical to manually-initiated transmit.

The same countdown screen and completion message will appear.

The main differences are 1) it happens under remote control

(no buttons hit), and 2) the sample number in the outbound

dump header is always that of the requested sample - no

substitutions occur.

See also the section below entitled "Sample Transmit -

Further Details" for more info on how the Studio 440

transmits samples.

========== Function Screen 3 - Requesting A Sample ==========

If you select "Request sample" in the SDS menu and then hit

the ENTER key, you'll see a screen which looks like this:

����������������Ŀ

�Receive into #12�

�req dump of # 12�

������������������

On the upper line you can select the sound number to which

the incoming sample will be assigned.  This is the number

of the "current" sound - thus you can select it by hitting

the pads, in addition to the other usual data entry methods.

However, the ALT PARAMS switch has no effect here.  Note that

if there is already a sound assigned to this number, it will

automatically be deleted when the incoming sample arrives

(unless the incoming sample is too large to fit into memory

and is rejected).

On the bottom line you indicate the number of the sample to

be requested from the other device.  This number tracks the

number in the top line, but can be changed to any value from

1 through 256, to allow the Studio 440 to work with samplers

which can have more than 32 samples.  It gets encoded into a

dump request message which is sent out by the Studio 440 when

you hit the ENTER switch.

After sending the dump request, the Studio 440 goes back to

standby.  It is up to the other sampler to respond with a

sample dump.  If it does, then receive operation is exactly

as described in the above section entitled "Remote Control

Operation", with one small but significant difference:

the Studio 440 will ignore the sample number which appears in

the incoming dump header (normally the number of the sample

you requested), and will store the sample under the Studio

440's current sound number (which you selected in the top

line of the request screen).  This happens regardless of the

setting of the "Rcv samp number" option, provided that the

other device responds within one second after the ENTER

switch is hit.  During this time, the Studio 440 assumes that

an incoming sample dump is a response to the dump request it

sent, and overrides the setting of this option - this is why

you will normally not need to change the setting of this

option.

If the other device does not respond, you will see:

����������������Ŀ

�  Request sent, �

�no response seen�

������������������

---- Function Screen 3 - Requesting a Sample (continued) ----

In addition to setup problems (recheck your MIDI cable

connections and MIDI channel settings), there are legitimate

reasons for failure to obtain a response:

1.  You may have requested a sample with a number beyond the

range which is recognized by the other sampler.

2.  The sample you requested may be deleted, empty, nil, etc.

Finally, there are some samplers which ignore the sample

number in a dump request and respond to the request by

sending whatever is their current sound.  When working with

one of these devices, you'll have to adjust the current sound

setting at both ends to achieve the desired transfer.

====================== Error Messages =======================

����������������Ŀ

�Sample too large�

�- unable to rcv �

������������������

The Studio 440 has received a dump header indicating a sample

size that is greater than the amount of sample memory the 440

can make available.  The incoming dump is not accepted.

����������������Ŀ

�Sample number is�

�  out of range  �

������������������

The Studio 440 has received a dump header with sample number

higher than 32, and can't store the sample as directed.

To get around this, use the "Request sample" function,

or change the "Rcv samp number" option from "dump header" to

"current snd" and hand-select the Studio 440 target sound

number to achieve the same result.

����������������Ŀ

�MIDI dump halted�

�by inapprop data�

������������������

The Studio 440 received a dump header or other SDS message

with invalid parameters or incorrect format, or received an

unexpected MIDI message of some other type while receiving a

sample dump.

����������������Ŀ

�Sample not there�

� - can't dump it�

������������������

An attempt has been made (either manually or via a dump

request message) to cause the Studio 440 to dump a deleted

sample.

����������������Ŀ

�MIDI dump halted�

�by you, the user�

������������������

User hit the STOP switch during sample send or receive -

sample transfer was stopped.

---------------- Error Messages (continued) -----------------

����������������Ŀ

�MIDI dump halted�

�by the receiver �

������������������

The Studio 440 received a CANCEL message from the other

device while sending a sample to it.  This often means that

the other device did not have enough memory available to

accept the sample being sent.  Some samplers send this

message normally after receiving a sample.  Some computers

use it as a method of obtaining dump headers only, in order

to find out what is currently in a sampler's memory.

����������������Ŀ

�  Request sent, �

�no response seen�

������������������

The Studio 440 sent a sample dump request as directed, but

has not seen a response to it (ie, an incoming dump header).

Still with me?  You're pretty dedicated.  Pretty serious.

Here are some more details you may find interesting.

============= Sample Receive - Further Details ==============

Upon recognizing an incoming sample dump header, the MIDI

response of the Studio 440 is always virtually immediate -

either a CANCEL message if the sample can't be accepted, or a

WAIT message to allow time for preparations to be made to

accept the sample.  If the preparations include deleting an

existing sample, there may be up to several seconds delay

before the Studio 440 is ready to accept sample data.

However, most SDS devices (including the Studio 440) will

wait only two seconds after sending a dump header before they

begin sending sample data, UNLESS they get a WAIT response

back from the receiving device before then.  For this reason,

the two-cable MIDI connection is always recommended.

Basic SDS provides for the transfer of a minimal amount of

info beyond the raw sample data itself.  Here's what the

Studio 440 does with this info:

1.  Sample rate.  Used to set the sample RATE of the received

sample.  Rates below 20KHz get the 16KHz setting, rates

between 20KHz and 35KHz get the 31KHz setting, and rates

above 35KHz get the 42KHz setting.

2.  Loop points.  Only one set is transmitted - the same LOOP

values are stored in both non-ALT and ALT parameter sets.

3.  Loop type.  SDS can indicate only forwards-only and

backwards-forwards loops, or no loop.  The Studio 440

uses the indicated loop TYPE, and sets up release loops.

One special case:  if the received loop start and end

points are equal, the loop TYPE is set to "LOOP OFF".

All other sound parameters are set to default values exactly

as though the sample had been created from scratch.  The

default name "MIDIdata" is given to samples received over

MIDI.

============= Sample Transmit - Further Details =============

The Studio 440 will not respond to a dump request with a

sample number higher than 32.  It will also ignore a dump

request if the requested sample is deleted.  When a dump

request is acknowledged and responded to, the sample number

received in the dump request message is set as the new

current sound before transmission begins.  MIDI response to a

dump request, if any, is always immediate.

When a sample is transmitted by the Studio 440 - in response

to either manual initiation or a received dump request - the

current sound and ALT PARAMS settings dictate what gets sent.

The currently active START and END points delimit the

"interesting" portion of the sample.  However, unlike the

Studio 440, the SDS protocol does not provide for the

transmission of START and END parameters - these are assumed

to be the beginning and end of the transmitted sample.

Because of this, the Studio 440 transmits samples starting

from the START point.  This is used as the "zero" point, even

though it may be set to some value other than zero in the

Studio 440.  Sample transmission ends at the END point, and

the LOOP points are sent with values which are relative to

the START point.  Thus, whatever truncation you have used on

a sound is preserved when the sound is transmitted.

Because a sound may have different START and END points, LOOP

points and/or LOOP TYPEs in its two parameter sets, it is

important to pay attention to the ALT PARAMS setting when

sending samples from the Studio 440.

The LOOP TYPE value used by the Studio 440 in the outgoing

dump header to indicate LOOP OFF is a recent addition to the

Sample Dump Standard which is not recognized by all samplers.

In order to be compatible with these devices, the Studio 440

sends both loop points equal to the end of the sample when

the sound being sent has a LOOP TYPE of LOOP OFF.

------------------------ That's it! -------------------------